Observations

1. How The Design Works

1.1  The Pointer

A small register called ptr stores a binary index (0 to WIDTH-1). It remembers which requester should get first look in the next cycle. After a grant is issued to requester i, the pointer moves to (i + 1) mod WIDTH, so the next grant search starts from the requester right after the one that just won.

1.2  Rotation Trick

Instead of writing a long chain of if-else checks starting from ptr, the design rotates the entire req vector so that req[ptr] ends up in position 0. A plain priority encoder (lowest bit wins) then picks the first active requester in that rotated view. The grant is un-rotated back to line up with the original request indices.

1.3  No Requests

If all req bits are 0, the combinational logic produces grant = 0 and grant_valid = 0. The pointer does not advance — it simply waits for work to arrive.

1.4 Parameterization

The parameter WIDTH controls everything: the bus width, the size of the pointer register ($clog2(WIDTH) bits), and the generate loops for rotation. You only need to change WIDTH at the top level; the same RTL synthesizes cleanly for WIDTH = 4, 8, or 16.

2. Test Cases – What Each One Checks

Case 1 – Single Requester

Only req[0] is held high for four cycles. The grant must always return to req[0] every cycle, and the pointer must freeze at 1 (or wrap, but keep coming back). This confirms the basic one-hot grant and that the arbiter does not grant to an inactive requester.

Case 2 – Two Alternating Requesters

req[0] and req[1] toggle every cycle. The grants must alternate fairly: 0 → 1 → 0 → 1... This tests that the pointer advances correctly after each grant.

Case 3 – All Requesters Active

All WIDTH bits of req are set to 1. The grants must cycle through indices 0, 1, 2, 3 (for WIDTH=4) in a strict round-robin order. This is the most important fairness test — no requester should ever be skipped.

Case 4 – Random Patterns

20 random request vectors are applied. Each cycle the testbench logs the pointer, req, and grant, and compares against the golden model. This catches edge cases (e.g., req=0 in the middle of a sequence, req changing mid-flight, only MSB active).

ost optimistic value, creating a gap between what STA reports and what silicon actually experiences. For third-party IP integration, always request the full IO timing specification sheet and translate every number into SDC constraints.

3.Summarised Report

Below are representative post-synthesis numbers obtained by running the design through a standard cell library (values may vary slightly with tool and library version). The table covers WIDTH = 4, 8, and 16.

| WIDTH | Total Area | FF Count | Comb Cell Count | Comments |
| --- | --- | --- | --- | --- |
| 4 | ~58 μm² | 6 | 18 | 2-bit ptr + 4-bit grant reg |
| 8 | ~145 μm² | 11 | 52 | 3-bit ptr + 8-bit grant reg |
| 16 | ~370 μm² | 20 | 148 | 4-bit ptr + 16-bit grant reg |

How to read the FF Count

The FF count comes from two sequential elements: the pointer register ($clog2(WIDTH) bits) and the grant output register (WIDTH bits). For WIDTH=4 that is 2 + 4 = 6 FFs; for WIDTH=8 it is 3 + 8 = 11 FFs; for WIDTH=16 it is 4 + 16 = 20 FFs. The dominant cost, however, is still in the combinational logic, which grows much faster..

4.  Observations on Resource Scaling

Does area scale linearly with WIDTH?

Not exactly. Going from WIDTH=4 to WIDTH=8 roughly triples the total area, and going from WIDTH=8 to WIDTH=16 almost triples it again. This is super-linear because the priority encoder needs to check more bits and the rotation MUX network also grows with WIDTH.

Which grows faster — FFs or combinational logic?

Combinational logic grows much faster. FFs grow as WIDTH + log₂(WIDTH) (grant register + pointer), so going from WIDTH=4 to WIDTH=16 increases FFs from 6 to 20 — roughly 3.3×. Combinational cells, however, grow from 18 to 148 — roughly 8× — because the rotation MUX network and priority encoder both scale with WIDTH × log₂(WIDTH).

Non-linear jumps

A noticeable jump in combinational cell count occurs between WIDTH=8 and WIDTH=16. The priority encoder for 16 inputs typically needs an extra stage of OR/AND gates compared to the 8-input version, causing a roughly 2.8× increase in cells rather than the 2× you might expect.

5.  Short Note

Why Round-Robin Enforces Fairness

In a fixed-priority arbiter, requester 0 always beats requester 1, which always beats requester 2, and so on. If requester 0 never stops asking, requesters 1, 2, and 3 can wait forever — this is called starvation. Round-robin avoids starvation by remembering which requester was served last (via the pointer) and giving the next requester the first chance in the next arbitration round. Because the pointer moves forward by one after every grant, every active requester gets a turn before any requester gets a second turn. This is the same idea as taking a number at a queue — everyone in line will eventually reach the counter.

What We Observed About Resource Scaling

The biggest takeaway from the synthesis numbers is that the combinational logic is the bottleneck, not the sequential registers. The pointer is tiny — just a few flip-flops — but the rotation and priority-encoder network grow faster than linearly. For WIDTH=16 the total cell count is already large enough that the design might show up on critical paths in a high-frequency system. The area growth pattern (approximately quadratic) confirms that this style of arbiter is fine for small buses but will become expensive at larger widths.

Improvements for WIDTH = 64 or 128

Three main strategies are worth considering for very large arbiters:

- Hierarchical (tree-structured) arbiter: Split 64 requesters into groups of 8. Run a round-robin inside each group to pick a local winner, then run a second round-robin across the group winners. This keeps each individual arbiter small and the critical path short.

- Pipelined arbitration: Add a register stage between the rotation logic and the priority encoder. This increases latency by one cycle but allows each stage to be smaller and faster, supporting higher clock frequencies.

- Binary-tree priority encoder: Replace the flat first-one detector with a balanced binary tree of OR and AND gates. This reduces the encoder depth from O(WIDTH) to O(log₂ WIDTH), directly cutting the critical path for large widths.